CISCO CCNA / PACKET TRACER LAB GUIDE

Configure Server-Based Authentication with TACACS+ and RADIUS

| TOPOLOGY COMPONENTS Routers R1, R2, R3 ¦ TACACS+ & RADIUS Servers ¦ PC-A, PC-B, PC-C | PROTOCOLS TESTED TACACS+ (Cisco Proprietary/TCP 49) & RADIUS (Open/UDP 1812/1645) |
| --- | --- |
| DEFAULT PRE-CONFIG CREDENTIALS Enable Secret: ciscoenpa55 ¦ OSPF MD5 Key: MD5pa55 | PRIMARY OBJECTIVES Configure AAA Model, Server Keys, Fallback Database, and Console Lines |

📌 LAB SCENARIO & OBJECTIVES: Currently, administrative security on routers R1, R2, and R3 relies solely on a shared enable secret password. In this lab, you will configure centralized Authentication, Authorization, and Accounting (AAA). Router R2 will authenticate administrators against a dedicated TACACS+ server, while Router R3 will authenticate against a RADIUS server. Both routers will be configured with a local database fallback in case the external AAA server becomes unreachable.

Network Addressing & Interconnect Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A | S1 F0/1 |
| R1 | S0/0/0 (DCE) | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | N/A | S2 F0/2 |
| R2 | S0/0/0 | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | S0/0/1 (DCE) | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 192.168.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
| R3 | S0/0/1 | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| TACACS+ Server | NIC | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 | S2 F0/6 |
| RADIUS Server | NIC | 192.168.3.2 | 255.255.255.0 | 192.168.3.1 | S3 F0/1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S1 F0/2 |
| PC-B | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S2 F0/1 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | S3 F0/18 |

⚠️ CONFIGURATION & COMPATIBILITY NOTES: The routers are pre-configured with enable secret password 'ciscoenpa55' and OSPF MD5 authentication key 'MD5pa55'. Note that Cisco IOS legacy commands (tacacs-server / radius-server) are used in Packet Tracer. In modern production IOS XE devices, replace these with 'tacacs server <name>' and 'radius server <name>'.

Part 1: Configure Server-Based AAA Authentication Using TACACS+ on R2

Step 1: Test Baseline Network Connectivity

• Open Command Prompt on PC-A and verify end-to-end IP routing by pinging PC-B and PC-C.

• Verify connectivity from PC-B to PC-C.

PC-A> ping 192.168.2.3 # Ping PC-B Sending 4, 32-byte ICMP Echos to 192.168.2.3: Success rate is 100% (4/4) PC-A> ping 192.168.3.3 # Ping PC-C Sending 4, 32-byte ICMP Echos to 192.168.3.3: Success rate is 100% (4/4) PC-B> ping 192.168.3.3 # Ping PC-C from PC-B Sending 4, 32-byte ICMP Echos to 192.168.3.3: Success rate is 100% (4/4)

Step 2: Configure a Backup Local Database User Account on R2

• Access Router R2 via console or CLI.

• Enter privileged EXEC mode using enable secret password 'ciscoenpa55'.

• Create a local fallback administrator account named Admin2 with secret password admin2pa55.

R2> enable Password: ciscoenpa55 R2# configure terminal R2(config)# username Admin2 secret admin2pa55

Step 3: Verify Pre-Configured TACACS+ Server Settings

• Click on TACACS+ Server in the Packet Tracer workspace.

• Navigate to the Services tab > select AAA.

• Verify the following pre-configured entries exist in the server database:

•   - Service: On

•   - Client Name: R2 | Client IP: 192.168.2.1 | Secret: tacacspa55 | Server Type: TACACS+

•   - User Setup: Username: Admin2 | Password: admin2pa55

Step 4: Define the TACACS+ Server and Pre-Shared Key on R2

• Specify the IP address of the TACACS+ Server (192.168.2.2).

• Configure the shared secret key 'tacacspa55' to authenticate R2 with the TACACS+ daemon.

R2(config)# tacacs-server host 192.168.2.2 R2(config)# tacacs-server key tacacspa55

Step 5: Enable AAA and Configure the Default Login Authentication Method

• Globally activate the AAA subsystem using aaa new-model.

• Define a default authentication login method list that first queries the TACACS+ group, and falls back to the local database if TACACS+ is unreachable.

R2(config)# aaa new-model R2(config)# aaa authentication login default group tacacs+ local

Step 6: Apply the AAA Authentication Method to the Console Line

• Enter console configuration mode (line con 0).

• Bind the console to the default AAA authentication method list using login authentication default.

R2(config)# line console 0 R2(config-line)# login authentication default R2(config-line)# exit R2(config)# exit

Step 7: Verify Server-Based TACACS+ Authentication on R2

• Exit privileged EXEC mode or press Enter on the console to initiate a new session.

• When prompted for Username and Password, enter the TACACS+ credentials (Admin2 / admin2pa55).

• Verify access is granted into R2 user EXEC mode.

R2# exit R2 con0 is now available Press RETURN to get started. User Access Verification Username: Admin2 Password: admin2pa55 R2> # Authenticated successfully via TACACS+ server!

Part 2: Configure Server-Based AAA Authentication Using RADIUS on R3

Step 1: Configure a Backup Local Database User Account on R3

• Access Router R3 CLI and enter privileged EXEC mode using password 'ciscoenpa55'.

• Create local fallback user Admin3 with secret password admin3pa55.

R3> enable Password: ciscoenpa55 R3# configure terminal R3(config)# username Admin3 secret admin3pa55

Step 2: Verify Pre-Configured RADIUS Server Settings

• Click on RADIUS Server in the Packet Tracer workspace.

• Navigate to the Services tab > select AAA.

• Confirm the following parameters are active:

•   - Service: On

•   - Client Name: R3 | Client IP: 192.168.3.1 | Secret: radiuspa55 | Server Type: RADIUS

•   - User Setup: Username: Admin3 | Password: admin3pa55

Step 3: Define the RADIUS Server and Pre-Shared Key on R3

• Configure the RADIUS server IP address (192.168.3.2) on R3.

• Configure the shared secret key 'radiuspa55'.

R3(config)# radius-server host 192.168.3.2 R3(config)# radius-server key radiuspa55

Step 4: Enable AAA and Configure the Default Login Authentication Method

• Activate the AAA subsystem using aaa new-model.

• Define the default login method list to authenticate using RADIUS first, followed by the local database fallback.

R3(config)# aaa new-model R3(config)# aaa authentication login default group radius local

Step 5: Apply the AAA Authentication Method to the Console Line

• Enter console configuration mode (line con 0).

• Bind console login to the default authentication list.

R3(config)# line console 0 R3(config-line)# login authentication default R3(config-line)# exit R3(config)# exit

Step 6: Verify Server-Based RADIUS Authentication on R3

• Exit privileged EXEC mode to lock the session.

• At the prompt, log in using RADIUS credentials: Username Admin3, Password admin3pa55.

• Confirm successful entry into user EXEC mode.

R3# exit R3 con0 is now available Press RETURN to get started. User Access Verification Username: Admin3 Password: admin3pa55 R3> # Authenticated successfully via RADIUS server!

Step 7: Check Results in Packet Tracer

• Click Check Results in the Packet Tracer task window.

• Confirm your completion score is 100%.

• Review the Assessment Items tab to ensure all AAA configurations, server keys, local database users, and console bindings are marked correct.

Key Differences: TACACS+ vs. RADIUS Comparison Table

| Feature / Characteristic | TACACS+ (Cisco Proprietary / RFC 8907) | RADIUS (Open Standard / RFC 2865) |
| --- | --- | --- |
| Transport Protocol & Port | TCP Port 49 (Connection-oriented, reliable) | UDP Port 1812 (Auth) & 1813 (Acct) [Legacy 1645/1646] |
| Packet Encryption | Encrypts entire packet payload (only header clear) | Encrypts password only; remainder of packet in plaintext |
| AAA Architecture | Separates Authentication, Authorization, & Accounting | Combines Authentication and Authorization together |
| Command Authorization | Granular per-command authorization support | Limited; typically restricts per-service or privilege level |
| Primary Use Case | Device administration & router/switch management | Network access (802.1X, VPNs, dial-up, captive portal) |

📌 LAB COMPLETION SUMMARY: You have successfully configured and verified centralized AAA authentication using both TACACS+ (on R2) and RADIUS (on R3), backed by local administrative fallbacks on both routers. Verify that your Packet Tracer completion score displays 100%.